- Introduction

The many-core processor based on the RC64 architecture is designed to provide high parallel computing capabilities for high-performance applications, especially in areas such as signal processing, communications, image processing, real-time applications, and space systems that require high reliability. The architecture is based on a combination of a large number of DSP cores operating in parallel, a multi-bank shared memory, and an internal communication network with a logarithmic structure that allows for low access times and a relatively low level of conflicts. This combination allows for a high performance-to-power ratio, while supporting hundreds or even thousands of short, continuous tasks.

At the heart of the architecture is the hardware scheduler block, which is the central component that coordinates all cores and their tasks. The role of this block is to manage the allocation, synchronization, and lifecycle of tasks in the system, ensure high core utilization, and maintain a stable workflow even under high computational loads. The scheduler is responsible for identifying which tasks are available for execution, assigning them to free cores in minimal time, tracking dependencies between tasks, and managing the termination of tasks and the execution of subsequent tasks. To achieve this, the scheduler is primarily divided into dedicated sub-modules: the Task Management Table (TMT) and the Core Management Table (CMT). 

The main advantage of the scheduler block is its ability to perform dynamic task management with extremely low response times, so that the system can support a programming model based on short task queues, high parallelism, and fast communication between different computational stages. This expansion of hardware capabilities allows system-level applications to enjoy high performance without the overhead of Operating System (OS) mechanisms, and without compromising the determinism required in real-time applications.

The scheduler block is a vital infrastructure component in the architecture, and defining its requirements, interfaces, and capabilities is a prerequisite for system stability and overall chip performance. This specification forms the basis for the logical design, verification, and integration of the scheduler into the entire many-core system.

2. System level - an overview

The many-core system based on the RC64 architecture is built around an array of 64 DSP cores operating in parallel, connected to a multi-bank shared memory system via a logarithmic network-on-chip (NoC). The system also includes DMA engines and hardware accelerators, which are managed as schedulable resources. This architecture enables efficient memory access, traffic reduction, and natural collision handling through built-in retry mechanisms. The cores do not communicate with each other directly; rather, they interact only through the shared memory and under the supervision of the scheduler. This allows for centralized control over the order of task execution and maintains the logical consistency of the computational process.

Within this architecture, the scheduler block plays a pivotal role in data flow and system behavior control. The scheduler receives descriptors of all tasks in the system, including their dependencies, and continuously maintains internal tables reflecting their states (Pending, Ready, Allocated, Terminated) as well as the status of each core (Idle / Busy). Based on this information, the scheduler assigns tasks to free cores, dispatches the execution parameters to them, and updates the task statuses accordingly. Upon task completion, the core reports back to the scheduler, which then evaluates whether dependent tasks have become ready, thereby sustaining the scheduling cycle.

Regarding data flow, the scheduler does not handle the computation payload (data path) itself, but strictly the control path. It acts as a management layer that directs how the cores access memory and the precise sequence of execution. Thus, the scheduler operates at the heart of the architecture, serving as a central command unit that dictates the pace and pipeline of the cores' workload.

To maximize system throughput, the scheduler must meet stringent latency requirements: the turnaround time between the completion of a task and the dispatch of the next task to a core must be minimal to prevent core starvation (idle cycles). This is critically important for the short tasks characteristic of the RC64 architecture, where even minor scheduling delays can significantly degrade overall system performance.

Key responsibilities of the system scheduler include:

- Dispatching both Regular Tasks (single instance) and Duplicable Tasks (data-parallel instances) to maximize core utilization.

- Managing dependencies between different processing stages to ensure a deterministic execution order.

- Handling hardware interrupts via pre-emptive High Priority Tasks (HPTs), ensuring immediate latency-bound responses for critical I/O operations.

- Maintaining system stability, determinism, and high execution efficiency across all operational scenarios.

In summary, the scheduler block bridges the logical requirements of the software with the physical processing capabilities of the silicon, making it one of the most critical components for the success of the RC64 many-core system.

3. Functional Requirements

### The Scheduler block acts as the central control unit for the RC64 many-core architecture. It is responsible for the dynamic mapping of the software task graph onto the physical core array. The following sections detail the required functionality.

#### 3.1. Task Lifecycle Management

### The scheduler shall implement a Finite State Machine (FSM) for every task in the system, supporting the following states:

- Pending (00): The task depends on predecessors and is not yet executable.

- Ready (01): All predecessors have completed; the task is available for allocation.

- Fully Allocated (10): All required instances of the task have been dispatched to the cores.

- Terminated (11): All instances have successfully completed execution.

### The scheduler shall enforce Task Dependencies by monitoring termination hardware signals. Upon the termination of a task, the scheduler must resolve the dependencies of successor tasks and transition them from the Pending to the Ready state if all conditions are met.

#### 3.2. Supported Task Types

### The scheduler logic must distinguish between and support two primary types of tasks:

### Regular Sequential Tasks: Single-instance tasks that run on a single core.

### Duplicable Tasks (Data Parallelism): Tasks capable of spawning multiple independent instances. The scheduler must manage a Duplication Quota for these tasks and allocate instances dynamically until the quota is exhausted.

#### 3.3. High Priority Tasks (HPT)

### To support strict real-time requirements and interrupt handling, the scheduler shall implement a pre-emptive mechanism for High Priority Tasks (HPTs):

### HPTs shall bypass the standard dependency checks and be dispatched immediately upon request (e.g., triggered by DMA interrupts).

### The scheduler must support up to 64 concurrent HPTs (one designated slot per core).

#### 3.4. Internal Data Structures

### To maintain the system state efficiently, the scheduler block shall implement and manage two primary hardware structures:

### A. Task Management Table (TMT) The TMT consists of 16 active rows to support high data parallelism. For each task entry, the table must store:

### Task ID and Duplication Quota (total number of instances required).

### Dependencies: A 16-bit row within the dependency matrix. Upon ingress, the incoming 10-bit blocking Task ID is evaluated against the existing 16 rows using combinational comparators. If a match is found, the corresponding bit in the matrix is asserted ('1').

### State (current status in the FSM lifecycle).

### Allocated Instances Counter (tracking the number of dispatched instances).

### Terminated Instances Counter (tracking the number of completed instances).

### B. Core Management Table (CMT) For each of the 64 cores, the table must store:

### Core State: A 64-bit status register tracking Idle/Busy states, coupled with a combinational priority encoder for rapid allocation.

### Assigned Task ID.

- Assigned TMT Row Index

### Assigned Instance ID (applicable for duplicable tasks).

#### 3.5. Performance and Latency Requirements

### Latency: The scheduler shall minimize pipeline "bubbles" (idle cycles) in the core execution units. The target latency between a core becoming Idle and receiving a new task allocation shall be strictly minimized (target: 1-3 clock cycles, relying on direct signal paths rather than network routing).

### Throughput: The scheduler logic must be capable of sustaining one task allocation and one task termination per clock cycle.

### Arbitration: In scenarios where multiple cores complete execution simultaneously, the Core Management Table (CMT) must arbitrate the signals and serialize the termination updates to the TMT (one per cycle) without losing data or stalling the execution pipeline.

#### 3.6. Error Handling and Recovery

### The scheduler must include hardware logic to detect and report the following exception scenarios:

### Invalid Termination: A core reports a termination for a task it was not assigned.

### Dependency Deadlock: Detection of cyclic dependencies or tasks trapped in the Pending state indefinitely.

### Recovery: The block shall support a "Safe Mode" or a partial reset capability to isolate faulty tasks without resetting the entire chip, thereby facilitating FDIR (Fault Detection, Isolation, and Recovery) mechanisms.

#### 3.7. Reset and Initialization

### Upon the assertion of the global RESET signal:

### All Core Table entries shall be initialized to the Idle state.

### All Task Table counters, matrices, and state registers shall be explicitly cleared.

### The scheduler shall be structurally ready to accept the initial "Boot Task" or load the primary task graph immediately following the de-assertion of the reset signal.

### 4. Block Interfaces

The RC64 Hardware Scheduler interacts with the system through dedicated, direct-wired interfaces, ensuring zero-latency decoupling between the scheduling logic (TMT/CMT) and the physical core array.

#### 4.1. Task Ingress Interface (FIFO)

This is the primary input channel for new tasks entering the scheduler. It operates as a direct interface with an upstream FIFO buffer.

- Mechanism: The TMT monitors the FIFO's empty flag. When space is available in the internal Task Register File (TRF), the scheduler asserts an acknowledge signal (TMT_FIFO_ack) to pop the next task payload via the 64-bit fifo_tmt_data bus. This payload directly feeds the scheduling matrix and contains the 32-bit TASK_ADDRESS, 10-bit TASK_ID, 10-bit DUPLICATE quota, and the 10-bit blocking DEPENDENCIES ID.

#### 4.2. Core Dispatch and Termination Interface

This interface replaces traditional network topologies with a direct, discrete signaling mechanism between the scheduler and the 64 cores, managed collaboratively by the TMT and CMT.

- Downstream (Dispatch): When the CMT identifies an Idle core (asserting ava_core_valid and providing the ava_core_id) and the TMT has a Ready task, the scheduler routes the instruction pointer via dispatch_addr and selects the target core via dispatch_core_id. This guarantees that the specific core receives its execution pointer immediately, in a single clock cycle.

- Upstream (Termination): Cores report completion by asserting their respective bit in the 64-bit core_done_vec and holding it high until acknowledged. The CMT's combinational priority encoder captures these signals, arbitrates them (processing one per cycle), and frees the core's status. It asserts a done_ack to the specific core (allowing it to lower its signal) and forwards a task_done_pulse alongside the terminated_tmt_idx to the TMT. The TMT then decrements the active instance counters and resolves downstream dependencies.

#### 4.3. Event and Interrupt Interface

To support the Pre-emptive High Priority Task (HPT) mechanism, the scheduler exposes an asynchronous event interface.

- This interface receives low-latency interrupt requests directly from system peripherals (e.g., DMA Controllers) and external triggers.

- Upon assertion of a signal on this interface, the scheduler immediately bypasses standard dependencies to allocate and dispatch the corresponding HPT, ensuring real-time responsiveness.

#### 4.4. Configuration and Control Interface

A standard memory-mapped slave interface (e.g., AXI4-Lite or APB) is provided for system administration:

- Initialization: Facilitates the loading of the initial Task Graph and configuration tables by the designated Boot Core.

- Debug & Monitoring: Provides read-access to internal hardware counters and state registers, enabling internal observability for FDIR (Fault Detection, Isolation, and Recovery) mechanisms.

- Host Control: Permits an external host controller to force state transitions or assert a soft reset to the block.

### 5. Block Interfaces and Protocols

The Scheduler block interacts with the system through dedicated hardware interfaces, replacing the legacy S-NoC packet-based topology with a direct, discrete signal handshake mechanism. This ensures zero-latency coupling between the TMT, CMT, and the execution units.

5.1. Signals and Buses

| Signal Name | Direction | Width | Description | Comments |
| --- | --- | --- | --- | --- |
| clk | Input | 1 | System Clock |  |
| rst_n | Input | 1 | Active Low Reset | Initializes FSMs and Matrices to IDLE. |
| FIFO Interface |  |  |  |  |
| fifo_tmt_data | Input | 64 | Task Data Payload | Includes 32-bit Address, 10-bit Global ID, 10-bit Run Count, and 10-bit Dependency ID. |
| empty | Input | 1 | FIFO Empty Flag | High when no new tasks are available. |
| tmt_fifo_ack | Output | 1 | FIFO Acknowledge | Asserts to pop the next task from the FIFO. |
| CORES Interface |  |  |  |  |
| dispatch_addr | Output | 32 | Task Instruction Address | Routed directly to the assigned core. |
| dispatch_core_id | Output | 6 | Target Core ID | Selects which of the 64 cores receives the address. |
| core_done_vec | Input | 64 | Core Termination Vector | Asserts when cores finish execution (held high until ack). |
| core_id_cmt_core | Output | 6 | Acknowledged Core ID | ID of the core whose termination is being acknowledged. |
| done_ack | Output | 1 | Termination Acknowledge | Signals the specific core to lower its done flag. |
| CMT Interface (Core Management) |  |  |  |  |
| ava_core_valid | Input | 1 | Core Available | Asserts when CMT has found an Idle core. |
| ava_core_id | Input | 6 | Available Core ID | The ID of the Idle core found by the CMT. |
| tmt_cmt_ack | Output | 1 | Allocation Acknowledge | TMT confirms a task was dispatched to the core. |
| task_id_tmt_cmt | Output | 10 | Dispatched Task ID | The Global Task ID assigned to the core. |
| tmt_idx_tmt_cmt | Output | 4 | Dispatched task index | Internal TMT row index (0-15) assigned to the core. |
| terminated_tmt_idx | Input | 4 | Terminated task index | Internal TMT row index of the completed task. |
| task_done_pulse | Input | 1 | Task Termination | Asserts for one cycle when a core's termination is processed. |
| terminated_task_id | Input | 10 | Terminated Task ID | The Global Task ID that just completed. |
| Configuration / HPT & Status |  |  |  |  |
| cfg_bus | In/Out | - | Config Interface | AXI-Lite/APB for initial loading and debug. |
| irq_req / irq_ack | In/Out | 64 | HPT Interrupts | Pre-emptive triggering for High Priority Tasks. |
| err | Output | 2 | Hardware Error Code | 00=OK, 01=Deadlock, 10=Invalid Termination. |

5.2. Protocols and Timing Diagrams

### The Scheduler block utilizes synchronous Handshake protocols for data integrity. Data is considered transferred (latched) only on the rising edge of clk when the defined conditions are met.

5.2.1. Ingress Protocol (FIFO to TMT)

### This interface fetches new tasks from the upstream software queue.

### Condition: The TMT internal Free Slot Manager indicates a free slot is available.

### Action: If empty is '0', the TMT asserts tmt_fifo_ack for one clock cycle.

### Result: On the next clock edge, the payload on fifo_tmt_data is written into the TMT's Register File. Simultaneously, the incoming 10-bit Dependency ID is evaluated via combinational comparators against existing active tasks, and the dependency matrix is updated instantly.

5.2.2. Dispatch Protocol (TMT and CMT to CORES)

### This is a synchronous 3-way handshake between the TMT, CMT, and the core array.

### Request: The CMT asserts ava_core_valid and provides the target core on ava_core_id.

### Condition: The TMT checks its Ready Arbiter. If a task is ready (dependencies resolved and Dispatch_Left > 0), it proceeds.

### Action: The TMT outputs the task address on dispatch_addr and passes the core ID on dispatch_core_id. Simultaneously, it asserts tmt_cmt_ack and provides the tmt_idx_tmt_cmt.

### Result: The core begins execution. The TMT decrements the task's Dispatch_Left counter. The CMT marks the core as 'Busy'.

5.2.3. Termination Protocol (Egress)

### This protocol frees resources and resolves dependencies without requiring internal FIFOs.

### Assertion: When a core finishes, it holds its respective bit high in the core_done_vec. The CMT utilizes a combinational priority encoder to select one core per cycle. It then asserts task_done_pulse to the TMT and drives the completed task's ID onto terminated_task_id. Simultaneously, the CMT asserts done_ack along with core_id_cmt_core to signal the specific core to lower its done flag.

### Action: The TMT's ID Match Logic identifies the local index of the task and decrements its Done_Left counter.

### Resolution: If Done_Left reaches zero, the TMT clears the corresponding column in the Dependency Matrix, potentially triggering the Ready state for successor tasks.

6. Internal Micro-Architecture

### 6.1 Block Diagram Overview

### The Hardware Scheduler is implemented as a centralized, high-speed custom-logic controller designed to manage system resources strictly through discrete hardware tables and combinational logic. This ensures O(1) decision-making capabilities. The internal architecture eschews network topologies in favor of direct signaling, consisting of the following primary structures:

### A. Internal Data Structures

### Task Register File (TRF): The central repository for active and pending tasks (16 entries). It stores the Global Task ID, Instruction Address, and two dedicated counters: Dispatch_Left and Done_Left (initialized to the Duplication Quota).

### Dependency Matrix: A 2D array of flip-flops (16x16) mapping pending tasks (rows) to blocking tasks (columns). It utilizes combinational NOR-gates to instantly identify when a task's dependencies are resolved.

### Core Management Table (CMT) Register: A 64-bit status register tracking the real-time Idle/Busy state of all 64 DSP cores, coupled with a Priority Encoder for core selection.

### B. Logic and Processing Engines

### Free Slot Manager & FIFO Ingress: Monitors the TRF for available entries. When space is available, it acknowledges the upstream FIFO to pop new task metadata directly into the TRF. It includes 16 combinational comparators that match the incoming Dependency ID against existing TRF rows to update the Dependency Matrix instantly.

### Ready Arbiter (Allocation Logic): Continuously evaluates the Dependency Matrix outputs and Dispatch_Left counters. It selects one valid, ready task per cycle to dispatch to an Idle core identified by the CMT.

- Termination Engine: Upon receiving a termination signal and a 4-bit TMT index from the CMT, it acts as a direct decoder. It accesses the specific TRF row in O(1) directly via the index to decrement its Done_Left counter, completely avoiding associative matching.

### 6.2 Sub-Block Details

### 1. Configuration Interface

### Inputs/Outputs: cfg_addr, cfg_wdata, cfg_we, cfg_cs, cfg_rdata.

### Functionality: Acts as the primary slave interface. Allows the Boot Core to configure initial task graphs and provides read-access to internal counters and error registers for FDIR monitoring.

2. Task Management Table (TMT)

### Inputs: fifo_tmt_data, empty (FIFO); task_done_pulse, terminated_tmt_idx, ava_core_valid, ava_core_id (CMT).

### Outputs: tmt_fifo_ack, dispatch_addr, dispatch_core_id, task_id_tmt_cmt, tmt_idx_tmt_cmt, tmt_cmt_ack.

### Functionality: Houses the TRF and Dependency Matrix. Manages the active state of tasks. Decrements Dispatch_Left upon allocation and Done_Left upon termination. Clears task entries and dependency columns strictly when Done_Left == 0.

3. Core Management Table (CMT)

### Inputs: core_done_vec (64-bit from cores); task_id_tmt_cmt,tmt_idx_tmt_cmt tmt_cmt_ack (TMT).

### Outputs: ava_core_valid, ava_core_id, task_done_pulse, terminated_tmt_idx, core_id_cmt_core, done_ack.

### Functionality: Tracks the operational state of all 64 DSP cores. Incorporates a Priority Encoder to serialize simultaneous core terminations (forwarding them one per cycle to the TMT while sending done_ack back to the specific core) and selects the next available Idle core for new allocations.

4. HPT (High Priority Task Controller)

### Inputs/Outputs: irq_req [64-bit], irq_ack [64-bit].

### Functionality: Manages pre-emptive scheduling. It synchronizes asynchronous triggers from DMA/peripherals and injects the corresponding execution address directly to the cores, bypassing the TMT's standard dependency matrix.

6.3 Data Flow

### The data flow is strictly partitioned into three discrete hardware paths, operating concurrently.

### 6.3.1 Ingress Path (Task Load)

### Trigger: Upstream FIFO empty == 0 AND Free Slot Manager indicates a vacant TRF row.

### Action: TMT asserts tmt_fifo_ack. In the next clock cycle, task data (Address, ID, Dependency ID, Quota) is latched into the TRF. Dispatch_Left and Done_Left counters are initialized. Simultaneously, the 10-bit Dependency ID is compared against all active TRF rows to assert the correct bit in the 16x16 Dependency Matrix.

### 6.3.2 Downstream Data Flow (Task Allocation)

### Trigger: CMT asserts ava_core_valid (Idle core found) AND Ready Arbiter identifies a task (Dependencies cleared AND Dispatch_Left > 0).

### Action: TMT routes the task dispatch_addr to the cores, selects the core via dispatch_core_id, and asserts tmt_cmt_ack.

### Update: The allocated task's Dispatch_Left counter is decremented by 1. The CMT marks the selected core as 'Busy'.

### 6.3.3 Upstream Data Flow (Task Termination)

### Trigger: A core finishes execution and holds its bit in core_done_vec high.

### Match & Update: The CMT arbitrates the signals using a priority encoder and asserts task_done_pulse to the TMT alongside the completed terminated_tmt_idx. Simultaneously, it sends done_ack to the specific core to clear its termination flag. The TMT directly decodes the provided 4-bit index to access the specific TRF row and decrements Done_Left by 1.

### Dependency Trigger: If Done_Left reaches 0, the task's corresponding column in the Dependency Matrix is asynchronously cleared, instantly transitioning any dependent successor tasks to the 'Ready' state. The TRF row is marked invalid (freed).

### 6.4 Control Flow and Performance FSM

### 6.4.1 Task Lifecycle FSM

### Pending: Task is in TRF, but its NOR-gate in the Dependency Matrix evaluates to '0'.

### Ready: Dependency NOR-gate evaluates to '1' AND Dispatch_Left > 0.

### Fully Allocated: Dispatch_Left == 0, but Done_Left > 0. The task remains in the TRF awaiting final terminations but is ignored by the Ready Arbiter.

### Terminated: Done_Left == 0. The FSM instantly clears the row, releasing resources.

### 6.4.2 Core Status FSM (CMT)

### Idle: Bit in the 64-bit register is '0'. Eligible for the Priority Encoder.

### Busy: Bit is '1'. The core is actively executing.

6.5 Performance and Quality of Service (Q&S)

### The micro-architecture is rigorously optimized to meet real-time constraints without network overhead:

### Latency Targets: Due to the direct-wired combinational logic (Dependency Matrix and Arbiters), the target latency from a core becoming Idle to receiving a new Allocation is 1 to 3 clock cycles.

### Throughput and Arbitration: The TMT logic is capable of sustaining exactly one task allocation and one task termination per clock cycle. To prevent data loss when multiple cores finish concurrently, the CMT includes hardware arbitration to serialize incoming terminations without the need for internal buffers.

### Real-Time Q&S: HPTs are granted absolute priority and bypass the TMT logic entirely, ensuring deterministic and immediate interrupt servicing for mission-critical events.

7. Register Map

### The Hardware Scheduler is programmed and monitored via a standard 32-bit slave configuration interface (e.g., AXI4-Lite). The address space is divided into Global Control/Status Registers, the Task Management Table (TRF entries), and the Core Management Table (CMT entries).

7.1 Register Summary

### Note: Address offsets are based on 32-bit word alignment.

| Register Name | Address Offset | Width | Access | Description |
| --- | --- | --- | --- | --- |
| SCHED_CTRL | 0x000 | 32 | R/W | Global Control: Reset, Enable, Safe Mode |
| SCHED_STATUS | 0x004 | 32 | RO | Global Status: Error codes (FDIR), Busy flags |
| HPT_IRQ_VEC_L | 0x008 | 32 | RO | Active Interrupt Request vector (Cores 0-31) |
| HPT_IRQ_VEC_H | 0x00C | 32 | RO | Active Interrupt Request vector (Cores 32-63) |
| TMT_BASE | 0x100 - 0x1FF | 32 | R/W | Task Register File Area (Entries 0 to 15) |
| CMT_BASE | 0x200 - 0x2FF | 32 | RO | Core Management Table Area (Entries 0 to 63) |

7.2 Detailed Register Descriptions

7.2.1 SCHED_CTRL (Global Control Register - 0x000)

| Bit(s) | Name | R/W | Reset | Description |
| --- | --- | --- | --- | --- |
| 0 | SW_RST | R/W | 1'b0 | Software Reset: Writing '1' clears all internal TRF entries, matrices, and CMT states. |
| 1 | SCHED_EN | R/W | 1'b0 | Enable: Set to '1' to allow FIFO ingress and Core dispatching. |
| 2 | SAFE_MODE | R/W | 1'b0 | Safe Mode: When enabled, suspends hardware allocation and allows host to manually override TMT/CMT states for FDIR. |
| 31:3 | RESERVED | - | 0 | Reserved. |

7.2.2 SCHED_STATUS (Global Status & Error Register - 0x004)

| Bit(s) | Name | R/W | Reset | Description |
| --- | --- | --- | --- | --- |
| 3:0 | ERR_CODE | RO | 4'h0 | Error Status: 0: OK 1: Dependency Deadlock 2: Invalid Termination ID 3: FIFO Over/Underflow |
| 4 | TRF_FULL | RO | 1'b0 | TRF Full: High if all 16 slots in the Task Register File are occupied. |
| 5 | CMT_FULL | RO | 1'b0 | CMT Full: High if all 64 cores are currently busy. |
| 31:6 | RESERVED | - | 0 | Reserved. |

7.2.3 TMT_n_CFG (TRF Entry Configuration - Offset 0x100 + n*16)

| Bit(s) | Name | R/W | Reset | Description |
| --- | --- | --- | --- | --- |
| 9:0 | GLOBAL_ID | R/W | 10'h0 | Global Task ID: The 10-bit system-wide identifier for the task. |
| 19:10 | QUOTA | R/W | 10'h0 | Duplication Quota (Run Count): Number of instances to execute. |
| 20 | VALID | R/W | 1'b0 | Valid: '1' indicates this TRF slot is actively in use. |
| 31:21 | RESERVED | - | 0 | Reserved. |

7.2.4 TMT_n_ADDR (TRF Instruction Address - Offset 0x104 + n*16)

| Bit(s) | Name | R/W | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | T_ADDR | R/W | 32'h0 | Task Address: The instruction pointer sent to the core upon dispatch. |

7.2.5 TMT_n_STATE (TRF Dynamic State - Offset 0x108 + n*16)

| Bit(s) | Name | R/W | Reset | Description |
| --- | --- | --- | --- | --- |
| 9:0 | DISPATCH_LEFT | RO | 10'h0 | Allocations Remaining: Counts down from Quota to 0. |
| 19:10 | DONE_LEFT | RO | 10'h0 | Terminations Remaining: Counts down from Quota to 0. |
| 21:20 | FSM_STATE | RO | 2'b00 | Hardware State: 00: Pending 01: Ready 10: Fully Allocated 11: Terminated (transient) |
| 31:22 | RESERVED | - | 0 | Reserved. |

7.2.6 TMT_n_DEP (Dependency Matrix Row - Offset 0x10C + n*16)

| Bit(s) | Name | R/W | Reset | Description |
| --- | --- | --- | --- | --- |
| 15:0 | DEP_VECTOR | R/W | 16'h0 | Dependency Matrix Row: Each bit [0-15] represents a local TRF index. If bit 'k' is '1', this task (n) is blocked until task 'k' terminates. |
| 31:16 | RESERVED | - | 0 | Reserved. |

7.2.7 CMT_n_STAT (Core State - Offset 0x200 + n*4)

| Bit(s) | Name | R/W | Reset | Description |
| --- | --- | --- | --- | --- |
| 0 | CORE_BUSY | RO | 1'b0 | Busy Flag: 0: Idle, 1: Busy. |
| 10:1 | TASK_ID | RO | 10'h0 | Active Task ID: The Global Task ID currently executing on this core. |
| 15:11 | TMT_IDX | RO | 4’h0 | Task index in TMT. |
| 31:16 | RESERVED | - | 0 | Reserved. |

8. Debug, Verification & Physical Design

8.1 Physical Design Assumptions

### The Hardware Scheduler is a centralized control block implemented as purely combinational and sequential custom RTL logic to ensure deterministic performance and zero-overhead latency.

### Technology: 65nm CMOS using RadSafe™ Rad-Hard-By-Design (RHBD) technology.

### Target Frequency: Designed for a clock frequency of 300 MHz, fully synchronized with the DSP cores and the local configuration buses.

### Reliability & Radiation Hardness:

### TID (Total Ionizing Dose) tolerance of 300 kRad(Si).

### Latch-up immunity and extremely low SEU (Single Event Upset) rates.

### Memory Protection: Internal hardware structures (the Task Register File, Dependency Matrix flip-flops, and CMT registers) utilize specialized RHBD techniques (e.g., TMR or localized parity/ECC) to protect against radiation-induced bit flips.

### Power Target: The block is optimized for low-power operation, utilizing clock gating where applicable, contributing to the overall chip target of less than 10 Watts.

8.2 Design Methodology

### Implementation: Developed as high-performance custom RTL logic. Due to the deep combinational paths inherent in the Dependency Matrix and Priority Encoders, rigorous Static Timing Analysis (STA) is mandated to meet the 300 MHz target.

### Development Tools: Integration with the RC64 SDK, utilizing the cycle-accurate parallel simulator for timing-accurate verification of task dispatching.

### Emulation: Use of the Many-Task Emulator (MTE) to validate the scheduling logic against complex, highly parallel software workloads during the pre-silicon design phase.

8.3 Verification Main Targets

### The verification environment (UVM/Formal) must explicitly cover and ensure the following:

### Functional Correctness: Seamless execution of complex Task Graphs, ensuring flawless dependency resolution via the matrix and strict quota management for duplicable tasks via the internal counters.

### Latency & Throughput: Validation that the scheduler maintains the strict 1 to 3 cycle core allocation latency, and that the CMT arbitration logic correctly serializes simultaneous core terminations without data loss or pipeline stalling.

### FDIR Logic: Verification of exception handling, specifically Dependency Deadlock detection, Invalid Termination reporting, and TRF/CMT overflow protections.

### Asynchronous Robustness: Ensuring the HPT Controller correctly synchronizes, pre-empts, and acknowledges external interrupt requests (irq_req) under all asynchronous corner cases.

8.4 Debug and Monitoring Features

### Hardware Observability: The AXI4-Lite Configuration Interface exposes the real-time status of the TRF (including the dynamic Dispatch_Left and Done_Left counters) and the CMT, providing deep observability for FDIR reporting.

### Trace Buffers: Supports the use of trace buffers in shared memory to record scheduling events (allocations and terminations) with exact timestamps, enabling software rollback analysis and performance tuning.

### Manual Overrides (Safe Mode): Via the SCHED_CTRL register, the design allows the system to enter a "Safe Mode." This suspends hardware scheduling and allows the host to inject "software events" to manually force task state transitions, isolate faulty cores, or flush deadlocked tasks for advanced debugging and in-flight recovery.

9. Assumptions and Open Issues

9.1 Design Assumptions

### The micro-architecture of the Hardware Scheduler is based on the following technical and system-level assumptions:

### Clock Frequency: The block is assumed to operate at a stable 300 MHz clock, strictly synchronized with the DSP cores and the local configuration buses.

### Performance Metrics: All performance figures, including the aggressive target latency of 1 to 3 cycles and an overall core utilization of 99%, are based on cycle-accurate RTL simulations and validation via the Many-Task Emulator (MTE).

### Configuration Authority: It is assumed that the Boot Core or an external host is solely responsible for the logical integrity and correctness of the Task Graph loaded into the upstream FIFO and TMT. The hardware scheduler executes the graph as provided and does not perform semantic validation of the software logic.

### Radiation Hardening: The physical design assumes the availability of the 65nm RadSafe™ library for the implementation of RHBD (Rad-Hard-By-Design) standard cells, particularly for the flip-flop heavy Dependency Matrix and TRF.

9.2 Open Issues and Future Improvements

### The following items remain under evaluation or are identified for future architectural iterations:

### Scalability and CAM Implementation: The current design utilizes parallel ID Match Logic (comparators) to emulate Content-Addressable Memory (CAM) behavior for a limited TRF depth (e.g., 16 tasks). Scaling the TRF to hold 64+ concurrent tasks will significantly increase combinational depth, requiring evaluation of physical true-CAM macros or pipelined matching stages to meet the 300 MHz timing closure.

### Termination Arbitration Bottlenecks: Further analysis is required to determine the architectural impact of massive simultaneous task terminations. Currently, the CMT serializes terminations (one per cycle). Extremely short, highly duplicated tasks might saturate this arbitration path, requiring future exploration of multi-issue termination logic.

### Hybrid Scheduling: Evaluation of a hybrid approach—where a dedicated, high-speed microcontroller core handles the heavy task-graph parsing while the hardware matrix manages real-time dispatching—is an open research item for next-generation system scalability.

### FDIR Protocol Refinement: The precise software-to-hardware handshake sequence for "Safe Mode" recovery during complex, multi-core cascading failures is subject to ongoing refinement based on mission-specific fault-tolerance profiles.